TerraTec

ActiveRadio

Win32 Application Programmers Interface

Version 2.90

Author: Friedhelm Birth

Date: 29.07.26

Contents

1	Introduction	2

1.1	Application Notes	3

1.1.1	Linking	3

1.1.2	Header file	3

1.1.3	Configuration file	3

1.1.4	Usage under Windows NT	3

2	Reference	4

2.1	Exported Functions	4

2.1.1	Overview	4

2.1.2	radioGetNumDevs	5

2.1.3	radioGetDevCaps	5

2.1.4	radioSelect	6

2.1.5	TerraTVInit	6

2.1.6	TerraTVExit	6

2.1.7	radioGetState	6

2.1.8	radioGetFrequency	6

2.1.9	radioSetFrequency	6

2.1.10	radioSearch	6

2.1.11	VolumeMute	6

2.1.12	VolumeSet	6

2.1.13	RDS_Start	6

2.1.14	RDS_Stop	6

2.1.15	RDS_Resync	6

2.1.16	RDS_EvalData	6

2.1.17	SAA6588_GetDecoderState	6

2.1.18	OM5610 tuner control functions	7

2.1.19	SAA6588 RDS/RBDS pre-processor control functions	11

2.1.20	Volume control functions	16

2.1.21	Multi-card support functions	19

2.2	Exported Event Handles	21

2.2.1	hEventNewGroup	21

2.2.2	hEventRT	21

2.2.3	hEventPS	21

2.2.4	hEventCT	21

2.2.5	Example	22

2.3	Type Definitions & Structures	23

2.3.1	General	23

2.3.2	RDS structures	26

Appendix	29

A.	PTY codes for RDS/RBDS	29

B.	Bibliography	30

# Introduction

The ActiveRadio is a FM tuner card with RDS. The hardware exist in two versions: As an ISA Standalone card and as an upgrade that can be connected to several TerraTec products. The ActiveRadio software and this API also work with the TerraTV Radio+ which uses a combined FM/TV tuner.

TerraTec provides a DLL “RADIO_IO.DLL”  to access the ActiveRadio hardware. The DLL exports functions for

- Hardware detection and initialization

- OM5610/FM1216 tuner control

- SAA6588 RDS/RBDS pre-processor control, basic RDS-decoding

- Volume control

A sample Visual C++ project is provided to show the basic usage of the API.

## Application Notes

### Linking

It is essential that the DLLMain function is called prior to all other function calls. This ensures that the ActiveRadio hardware is properly initialized. Therefore it is recommended to statically link the DLL using the provided LIB file. In this case DLLMain is called automatically.

### Header file

The include file “IOEXPORT.H” contains all necessary type definitions and export function declarations. Only the functions and structures mentioned in this reference are verified. All others may be temporarily and should not be used.

### Configuration file

The volume control for the ActiveRadio Upgrade module is achieved via the Windows mixer functions (Windows Sound System). The only exception is when ActiveRadio Upgrade is used with the TerraTV+, where mute on/off is controlled directly on the hardware.

New drivers for the carrier boards may lead to different string names and mixer control IDs. Therefore a INI file is supplied which contains important parameters for the hardware control. When you use the DLL you must supply the INI file (TTRADIO.INI) in your application directory!

### Usage under Windows NT

For Windows NT you have to install an additional driver. The easiest way to do that is to simply install the original ActiveRadio software. The driver is installed within this setup.

# Reference

## Exported Functions

### Overview

#### Initialization / Configuration

RDSLIBAPI int       radioGetNumDevs ();

RDSLIBAPI RDSRETURN radioGetDevCaps (int nIdx,RADIOCAPS* pCaps);

RDSLIBAPI RDSRETURN radioSelect     (int nIdx);

RDSLIBAPI void      TerraTVInit();

RDSLIBAPI void      TerraTVExit();

#### Tuner control

RDSLIBAPI RDSRETURN radioGetState    (TUNERSTATE* psState);

RDSLIBAPI RDSRETURN radioGetFrequency(PULONG lpdwFreq,TUNERSTATE* pTs);

RDSLIBAPI RDSRETURN radioSetFrequency( ULONG   dwFreq,TUNERSTATE* pTs);

RDSLIBAPI RDSRETURN radioSearch      (PULONG lpdwFreq,BOOL fDir,TUNERSTATE* pTs);

#### Volume control

RDSLIBAPI RDSRETURN VolumeMute(BOOL fMute);

RDSLIBAPI RDSRETURN VolumeSet(WORD wNewVolume);

#### RDS control

RDSLIBAPI RDSRETURN RDS_Start(WORD wReserved);

RDSLIBAPI RDSRETURN RDS_Stop();

RDSLIBAPI RDSRETURN RDS_Resync();

RDSLIBAPI RDSRETURN RDS_EvalData(RDS_DATA*);

RDSLIBAPI RDSRETURN SAA6588_GetDecoderState(SAA6588_STATE*);

### radioGetNumDevs

Purpose: Returns the number of ActiveRadio hardware instances.

### radioGetDevCaps

Purpose:  Returns information about the installed ActiveRadio hardware.

typedef struct tagRADIOCAPS

{

int             nIndex;

TTRADIO_HWID    HardwareId;

TTRADIO_HWTYPE  HardwareType;

TTRADIO_BUSTYPE BusType;

char            szDesc[128];  // description

DWORD           dwCaps;

}RADIOCAPS;

// Hardware IDs

typedef enum {

TTRADIO_INVALID_HARDWARE = -1,

TTRADIO_TUNER_STANDALONE =  0,     // Radio-Boostar

TTRADIO_TUNER_UPGRADE    =  1,     // Sound-Boostar

TTRADIO_RDS_STANDALONE   =  2,     // TerraTec ActiveRadio Standalone

TTRADIO_BASE1            =  3,     // TerraTec Base1/64

TTRADIO_MANUAL           =  4,     // internal use only

TTRADIO_XLERATE          =  5,     // TerraTec XLerate

TTRADIO_EWS64S           =  6,     // TerraTec EWS64S

TTRADIO_TERRATV          =  7,     // TerraTec TerraTV+

(running on VxD drivers)

TTRADIO_DMX              =  8,     // TerraTec DMX

TTRADIO_TERRATV_RADIO    =  9,     // TerraTec TerraTV Radio+

(running on VxD drivers)

TTRADIO_TERRATV_RADIO_WDM= 10,     // TerraTec TerraTV Radio+

(running on WDM drivers)

TTRADIO_TERRATV_WDM      = 11,     // TerraTec TerraTV+

(running on WDM drivers)

TTRADIO_TERRATV_RADIO_WDM_MONO = 12// TV Radio mono, Animation

(running on WDM drivers)

}TTRADIO_HWID;

typedef enum

{

TTRADIO_TYPE_STANDALONE,

TTRADIO_TYPE_UPGRADE,

TTRADIO_TYPE_TVTUNER,

TTRADIO_NUM_TYPES

}TTRADIO_HWTYPE;

typedef enum

{

TTRADIO_BUS_ISA,

TTRADIO_BUS_PCI,

TTRADIO_BUS_USB,

TTRADIO_BUS_IEEE1394,

TTRADIO_NUM_BUSES

}TTRADIO_BUSTYPE;

// dwCaps

#define TTRADIO_HAS_RDS           0x00000001

#define TTRADIO_HAS_HW_VOLUME     0x00000002

#define TTRADIO_HAS_HW_MUTE       0x00000004

#define TTRADIO_IS_MONO           0x00000008

#define TTRADIO_TV_IS_WDM         0x00001000 // TV cards only: uses WDM drivers (as opposed to VxD)

#define TTRADIO_TV_SHARES_TUNER   0x00002000 // TV cards only: combined TV/FM TUNER

#define TTRADIO_TV_SHARES_VOLUME  0x00004000 // TV cards only:

TV and FM uses the same volume switches

### radioSelect

### TerraTVInit

Must be called before any other function if you want enumeraste and use TerraTec TV cards running under WDM drivers.

### TerraTVExit

If you called TerraTVInit( ) call this function before your program quits.

### radioGetState

### radioGetFrequency

### radioSetFrequency

### radioSearch

### VolumeMute

### VolumeSet

### RDS_Start

### RDS_Stop

### RDS_Resync

### RDS_EvalData

### SAA6588_GetDecoderState

### OM5610 tuner control functions

The following functions control the tuner chip.

#### SetFrequency

The SetFrequency function sets the specified frequency. It also switches between stereo and mono output and between local/DX mode (see OM5610 data sheet).

RDSRETURN SetFrequency(DWORD dwFreq,TUNERSTATE* psState);

##### Parameters

DWORD dwFreq	New Frequency in Hz *10.  The frequency can range from 87.5 KHz to 108 KHz. The parameter values must be in the range [875000 .. 1080000]. The frequency step width is 12.5 KHz.

TUNERSTATE* psState	Set psState->bits.StereoMode = 1 for stereo output or 0 for mono output

Set psState->bits.DX  = 1 for DX or 0 for local mode

All other members of the TUNERSTATE structure are ignored.

##### Return Value

T_OK	if successful

T_ERROR_INVALID_FREQUENCY	if the input frequency was invalid

#### GetFrequency

The GetFrequency function retrieves the current frequency and tuner status information.

RDSRETURN GetFrequency(LPDWORD lpdwFreq,TUNERSTATE*);

##### Parameters

LPDWORD lpdwFreq	Pointer to a DWORD that receives the frequency. The returned value is in the range [875000 .. 1080000]. (see SetFrequency)

TUNERSTATE* psState	Pointer to a TUNERSTATE structure that is filled with tuner status information.

psState->bits.StereoSignal is 1 if the broadcasted signal is stereo or 0 if it is mono.

psState->bits.AFC is 1 if the AFC locked and 0 if it not locked to the frequency

##### Return Value

T_OK	if successful

T_ERROR_INVALID_FREQUENCY	if the readback frequency was invalid

##### Remarks

The GetFrequency returns the current frequency only once after a tuner write operation (SetFrequency or Search). After a second read the frequency is invalid.

#### GetState

The GetState function retrieves the tuner and signal state.

RDSRETURN GetState(TUNERSTATE* psState);

##### Parameters

TUNERSTATE* psState	Pointer to a TUNERSTATE structure that is filled with tuner status information.

psState->bits.StereoSignal is 1 if the broadcasted signal is stereo or 0 if it is mono.

psState->bits.AFC is 1 if the AFC locked and 0 if it not locked to the frequency

##### Return Value

RDS_OK	if successful

RDS_ERROR	else

##### Remarks

This function can be used to periodically update a status display.

#### Search

The Search function starts a search for the next frequency on which a radio program is transmitted. It starts at the current frequency and searches up or down depending on the parameters.

RDSRETURN Search(LPDWORD lpdwFreq,BOOL fDir,TUNERSTATE* psState);

##### Parameters

LPDWORD lpdwFreq	Pointer to DWORD that contains the current frequency before the call and that receives the frequency value if a station was found.

BOOL fDir	Direction flag. 1 to search up, 0 to search down.

TUNERSTATE* psState	Pointer to a TUNERSTATE structure that must be filled with the search mode values before the search and that receives tuner status information after the search.

psState->bits.DX must be set to 1 for DX mode or 0 for local mode

psState->bits.Level must be set to the desired search level [0..3]

psState->bits.StereoSignal is set to 1 if the broadcasted signal is stereo or 0 if it is mono.

psState->bits.AFC is set to 1 if the AFC locked and 0 if it didn’t lock to the found frequency.

##### Return Value

T_OK	if successful

T_ERROR_TOO_CLOSE	if the same station was found again

T_ERROR_NO_STATION_FOUND if no station could be found

##### Remarks

The search level sets a minimum signal level that a radio signal must reach to be found. A value of 3 finds the least stations, a level of 0 finds most stations. Also notice that in local mode the input signal is attenuated by 20 dB. Therefore you’ll find less station in local mode than in DX mode.

If you start a search with a low level. It may happen that the frequency from which the search was started is returned as search result. In this case the functions returns with T_ERROR_TOO_CLOSE. You have to shift the start frequency a little bit and call Search again.

### SAA6588 RDS/RBDS pre-processor control functions

#### RDS_Start

The RDS_Start function starts the polling mode. The SAA6588 RDS pre-processor is polled for new data about every 20 ms. New data is decoded and stored in an internal buffer.

RDSRETURN RDS_Start(WORD wPollPeriod);

##### Parameters

WORD wPollPeriod	Ignored

##### Return Value

RDS_OK 	if successful.

RDS_ERROR 	else.

#### RDS_Stop

The RDS_Stop function stops the polling of the SAA6588 decoder.

RDSRETURN RDS_Stop();

##### Parameters

None

##### Return Value

RDS_OK 	if successful.

RDS_ERROR 	else.

##### Remarks

You should always call RDS_Stop before you set a new frequency or start a search. Otherwise the low level control codes for the OM6510 and the SAA6588 may interfere with each other.

Call RDS_Start afterwards to start the polling mode again.

#### RDS_Resync

The RDS_Resync function immediately terminates the actual synchronization and restarts a new synchronization search procedure.

RDSRETURN RDS_Resync();

##### Parameters

None

##### Return Value

RDS_OK 	if successful.

RDS_ERROR 	else.

##### Remarks

This function can be used for a weak RDS signal when synchronization is lost and shall quickly be restarted.

#### RDS_EvalData

The RDS_EvalData function transfers the internal RDS data buffer to the application.

RDSRETURN RDS_EvalData(RDS_DATA* psData);

##### Parameters

RDS_DATA* psData	Pointer to a RDS_DATA structure that receives the current RDS data.

##### Return Value

RDS_OK 	if successful.

RDS_ERROR 	else.

##### Remarks

An application should call this function periodically or as reaction to one of events described in section 2.1.

#### SAA6588_GetDecoderState

The SAA6588_GetDecoderState function retrieves data about the current SAA6588 reception and error count state.

RDSRETURN SAA6588_GetDecoderState(SAA6588_STATE* psState);

##### Parameters

SAA6588_STATE * psState	Pointer to a SAA6588_STATE structure that receives the current decoder state information.

##### Return Value

RDS_OK 	if successful.

RDS_ERROR 	else.

##### Remarks

This function can be used to get information about the quality of the RDS signal.

### Volume control functions

The ActiveRadio hardware exist in two versions: As an ISA Standalone card and as an upgrade that can be connected to several TerraTec products.

Only the ISA Standalone card has a volume control circuit on board. The volume for all others must be controlled via the sound card. The DLL does all the volume configuration internally, so you don’t have to bother about that. However, the configuration values are read from an INI file that is installed with the ActiveRadio Software (TTRADIO.INI).

Make sure you copy this file to your application directory!!!

#### VolumeMute

The VolumeMute function sets the radio output volume off or on!

RDSRETURN VolumeMute(BOOL fMute);

##### Parameters

BOOL fMute	1 to mute the volume, 1 to unmute

##### Return Value

RDS_OK 	if successful.

RDS_ERROR_NOT_SUPPORTED 	if a non-supported device was specified.

RDS_ERROR 	else.

##### Remarks

#### VolumeSet

The VolumeSet function controls the volume

RDSRETURN VolumeSet(WORD wNewVolume);

##### Parameters

WORD wNewVolume	The volume can range from 0 ..127

##### Return Value

RDS_OK 	if successful.

RDS_ERROR_NOT_SUPPORTED 	for an ActiveRadio Upgrade module on a TerraTV

RDS_ERROR 	else.

##### Remarks

The TerraTV+ and TerraTV Radio+ cards handles volume control differently. Mute on/off is controlled directly on the hardware. The function VolumeSet always sets the master volume of the first audio device found in the system.

#### VolumeRecordSelect

The VolumeRecordSelect function automatically selects the radio as input source for a recording via the Windows Sound System.

RDSRETURN VolumeRecordSelect(void);

##### Parameters

None

##### Return Value

RDS_OK 	if successful.

RDS_ERROR_NOT_SUPPORTED 	for an ActiveRadio Upgrade module on a TerraTV or an ActiveRadio ISA Standalone card

RDS_ERROR 	else.

##### Remarks

This function only applies to the ActiveRadio UpgradeModule. It has no effect on the ISA Standalone card.

### Multi-card support functions

#### GetNoOfCards

To query the number of ActiveRadio devices installed in the system.

RDSLIBAPI int GetNoOfCards();

##### Parameters

None

##### Return Value

The number of ActiveRadio devices installed in the system.

#### SelectCard

With SelectCard one ActiveRadio device can be chosen on which all subsequent API functions are processed.

If this function is not called all API functions address the device with index 0.

RDSLIBAPI RDSRETURN SelectCard   (int nIdx);

##### Parameters

int nIdx	Index of the desired card

##### Return Value

RDS_OK

RDS_ERROR

#### GetCardName

To query the description of a installed ActiveRadio device. The returned string contains the type of ActiveRadio

(Standalone/Upgrade) and the soundcard on which the Upgrade module is installed.

RDSLIBAPI RDSRETURN GetCardName(int nIdx, char* pszName, int nBufSize);

##### Parameters

int nIdx	The index of the card to query

char* pszName	Pointer to string buffer to receive the device name

int nBufSize	Size of string buffer

##### Return Value

RDS_OK

#### GetCardHardwareID

This function returns an unique ID for each ActiveRadio/Soundcard combination.

RDSLIBAPI int GetCardHardwareID (int nIdx);

##### Parameters

int nIdx	The index of the card to query

##### Return Value

0 = TUNER_STANDALONE	// Radio-Boostar

1 = TUNER_UPGRADE		// Sound-Boostar

2 = RDS_STANDALONE	// TerraTec ActiveRadio Standalone

3 = BASE1            		// TerraTec Base1/64

4 = MANUAL           		// internal use only

5 = XLERATE          		// TerraTec XLerate

6 = EWS64S           		// TerraTec EWS64S

7 = TERRATV          		// TerraTec TerraTV+

8 = DMX              		// TerraTec DMX

9 = TERRATV_RADIO    	// TerraTec TerraTV Radio+

## Exported Event Handles

Instead of periodically calling RDS_EvalData it is possible to be notified by events sent by the DLL. The application can use the exported event handles in connection with WaitForMultipleObjects function to react on a event signal. An example how to achieve that is given at the end of this section.

### hEventNewGroup

HANDLE hEventNewGroup;

This event is set by the DLL after a complete RDS group was received. The group is then decoded and the internal RDS data buffer is updated with the new information. Use RDS_EvalData to get the updated RDS data structure.

### hEventRT

HANDLE hEventRT;

This event is set when a new radio text line starts. The complete previous radio text string is stored in the RT_bak member of the RDS_DATA structure.

### hEventPS

HANDLE hEventPS;

This event is sent everytime the decoder received a part of the station name until all eight characters were transmitted.

### hEventCT

HANDLE hEventCT;

This event is set everytime the decoder received a RDS time signal. A time signal is usually sent once every minute. The decoded time is stored in the CT member of the RDS_DATA structure.

### Example

Enumerate your events.

#define	EXIT_EVENT			0

#define	RDS_NEW_GROUP_EVENT		1

#define	RDS_PS_EVENT			2

#define	RDS_CT_EVENT			3

#define	RDS_RT_EVENT			4

#define 	EVENT_NO			5	// number of events used

Declare an event buffer to hold all events that you want to evaluate.

HANDLE	hEventBuffer[EVENT_NO];

hEventBuffer[RDS_RDS_GROUP_EVENT] = hEventNewGroup;

hEventBuffer[RDS_PS_EVENT]		 = hEventPS;

hEventBuffer[RDS_CT_EVENT]		 = hEventCT;

hEventBuffer[RDS_RT_EVENT]		 = hEventRT;

hEventBuffer[EXIT_EVENT] 		 = CreateEvent(NULL,FALSE,FALSE,"AppExit");

Create a wait thread that waits for the events and launches an action when an event was signalled.

DWORD  dwThreadID;

HANDLE hThread = CreateThread(NULL,4096, (LPTHREAD_START_ROUTINE)NotifyThread,

this,0,&dwThreadID);

The thread routine should look something like this

void NotifyThread()

{

DWORD 	dwEventHandle;

BOOL	fAbort = FALSE;

do{

dwEventHandle = WaitForMultipleObjects(nEvents,hEventBuffer,FALSE,INFINITE);

if (dwEventHandle == 0xFFFFFFFF)

break; 			// error

switch (dwEventHandle - WAIT_OBJECT_0)

{

case RDS_BUFFER_FULL_EVENT: 	// new RDS group received

RDS_EvalData(pRDS);

DoSomething();

break;

case RDS_PS_EVENT:			// PS changed

RDS_EvalData(pRDS);

DoSomething();

break;

case RDS_RT_EVENT:			// RT change flag was transmitted

RDS_EvalData(pRDS);

DoSomething();

break;

case RDS_CT_EVENT: 			// new RDS time (CT)

DoSomething();

break;

case EXIT_EVENT:			// this event must be sent by the

fAbort = TRUE;		// application to stop the thread

}					// end of switch ()

}while(!fAbort);

ExitThread(0);

}

Stop the thread when you are done with it.

SetEvent(hEventBuffer[EXIT_EVENT]);

## Type Definitions & Structures

### General

#### RDS_DATA

This structure contains all data that is decoded in the DLL and available for an application.

Used by the RDS_EvalData function.

typedef struct tagRDS_DATA

{

BYTE           PTY;         // Program type              (5 bit)

BYTE           TP;          // Traffic program           (1 bit)

BYTE           TA;          // Traffic announcement      (1 bit)

BYTE           DI;          // Decoder control           (4 bit)

BYTE           DI_chk;      // Decoder control checkword (4 bit)

BYTE           M_S;         // Musik-/Sprache-Kennnung   (1 bit)

BYTE           RT_Flag;     // flag changes, if a new text line is sent (1 bit)

//   (unfortunately not used by all stations)

BYTE           No_AF;       // size of Alternative Frequencies (AF) list

(list entries)

DWORD*         pAF_List;    // pointer to AF list

//   (list values are in kHz*100 e.g. 875000 for 87.5 MHz)

RDS_CC         CC;          // country code

RDS_PIN        PIN;         // Programmbeitragskennung

RDS_TIME       CT;          // Time signal

RDS_EON        EON;         // EON

WORD           PI;          // PI-Code

char           PS[9];       // Station name              (8 chars)

char           RT[65];      // Radiotext                 (64 chars)

char           RT_bak[65];  // backup of last complete RT string Radiotext

char           RT_tmp[65];  // temporary RT buffer to check for identical RT lines

}RDS_DATA;

Note: The members EON, PIN are not fully tested. Don’t use them!

#### RDSRETURN

Most functions return a RDSRETURN value as error code.

typedef unsigned long RDSRETURN;

If a function was successful it returns RDS_OK and an erro code else.

#### SAA6588_STATE

This structure contains information on the decoder state. The function SAA6588_GetDecoderState fills this structure.

Used by the SAA65878_GetDecoderState function.

typedef union tagSAA6588_STATE

{

WORD State;

struct tagSAA_bits {

WORD	SQI	:4;	// signal quality 0 ..15	 (4 bit)

WORD	BEC	:7;	// block error count 0..128(6 bits)

WORD	DOFL	:1;	// data overflow		 (1 bit)

WORD	DAC	:2;	// data output mode 0.. 2	 (2 bit)

WORD   SYNC	:1;	// sync			 (1 bit)

} bits;

}SAA6588_STATE;

Note: The SQI reflects the signal quality value of the SAA6588. However, this value is not a reliable indicator for the signal strength. It shows good values even for very poor reception. In the ActiveRadio software a cumulated value of all above members is used to build a RDS signal quality value

#### TUNERSTATE

In the TUNERSTATE structure all values and flags regarding th OM5610 tuner chip are kept. AFC and StereoSignal are values that are read back from the OM5610, DX, StereoMode and Level are values that are written to the OM5610.

Used by: SetFrequency, GetFrequency, GetState, Search.

typedef union tagTunerState {

BYTE	State;

struct tagTSBits{

BYTE AFC		:1;

BYTE StereoSignal	:1;

BYTE DX		:1;

BYTE StereoMode	:1;

BYTE Level		:2;

}bits;

}TUNERSTATE;

### RDS structures

The structures in this section are only used within the RDS_DATA structure. They reflect how the RDS data buffer is organized.

#### Country code (CC)

typedef union tagRDS_CC {		// country code (group type 1)

WORD word;

struct tagCC_bits {

WORD ECC_LSB :4;

WORD ECC_MSB :4;

WORD PI_MSB 	:4;

}bits;

}RDS_CC;

#### Pin code (PIN)

typedef union tagRDS_PIN {

WORD word;

struct tagPIN_bits {

WORD minute 	:6;

WORD hour	:5;

WORD day	:5;

}bits;

}RDS_PIN;

#### Time signal (CT)

typedef struct tagRDS_TIME

{

BYTE		KW;			// 6 bit calendar week;

BYTE		year;			// 7 bit

BYTE		month;			// 4 bit

BYTE		day;			// 5 bit

BYTE		weekday;		// 3 bit

BYTE		hour;			// 5 bit

BYTE		minute;		// 6 bit

}RDS_TIME;

#### Linkage Set Number (LSN)

typedef union RDS_LSN		// Linkage Set Number used for EON

{

WORD word;

struct tagRDS_LSN

{

WORD LSN	:4; 	// Linkage Set Number

WORD ILS	:1;	// International Linkage Set Identifier

WORD NU	:1; 	// not used

WORD EG	:1;	// artverwandte Sendebereichscodes

WORD LA	:1;	// Link Actuator

}bits;

}RDS_LSN;

#### EON

typedef struct tagRDS_EON

{

char		PS_ON[9];

BYTE		PTY_ON;

BYTE		TA_ON;

BYTE		TP_ON;

BYTE		AF_ON[2];

RDS_PIN	PIN_ON;

RDS_LSN 	LSN;

BYTE		FREQ_TN;	// Abstimmfrequenz (this network)

BYTE		FM_AF[4];

BYTE		LF_MF;

}RDS_EON;

Appendix

- PTY codes for RDS/RBDS

One of the differences of RDS (Europe) and RBDS (USA) is the meaning of the PTY (program type) values. They are listed in the following table.

| PTY | RDS | RBDS |
| --- | --- | --- |
| 0 | --- |  |
| 1 | NEWS | NEWS |
| 2 | CURRENT AFFAIRS | INFORMATION |
| 3 | INFORMATION | SPORTS |
| 4 | SPORT | TALK |
| 5 | EDUCATION | ROCK |
| 6 | DRAMA | CLASSIC ROCK |
| 7 | CULTURE | ADULT HITS |
| 8 | SCIENCE | SOFT ROCK |
| 9 | VARIED | TOP40 |
| 10 | POP | COUNTRY |
| 11 | ROCK | OLDIES |
| 12 | M.O.R. | SOFT |
| 13 | LIGHT CLASSICS | NOSTALGIA |
| 14 | SERIOUS CLASSICS | JAZZ |
| 15 | OTHER MUSIC | CLASSICAL |
| 16 | not used | R&B |
| 17 | not used | SOFT R&B |
| 18 | not used | LANGUAGE |
| 19 | not used | RELIGIOUS MUSIC |
| 20 | not used | RELIGIOUS TALK |
| 21 | not used | PERSONALITY |
| 22 | not used | PUBLIC |
| 23 .. 29 | not used | not used |
| 30 |  | EMERGENTY TEST |
| 31 | ALARM | EMERGENCY |

Table 2-1: PTY tables

- Bibliography

Specifications

DIN EN50067, Spezikation des Radio-Daten-Systems (RDS), Beuth Verlag, Berlin, 1992

Data sheets

OM5610 Matchbox global FM tuner, Philips Semiconductors

SAA6588 RDS/RBDS pre-processor, Philips Semiconductors

FM1216 Desktop video & FM radio module, Philips Semiconductors, 1997

TerraTec Electronic GmbH

ActiveRadio - Hardware Interface

ActiveRadio - Windows NT 4.0 Driver Reference

TerraTec Products with a radio connector are: Base 1, Base 64, XLerate, EWS64 S, TerraTV+, DMX

Not using MFC!

The latest version of TTRADIO.INI is installed with the ActiveRadio software.

In local mode the radio signal is attenuated by about 20 dB

Automatic Frequency Control

Automatic Frequency Control

Automatic Frequency Control

TerraTec Products with a radio connector ar: Base 1, Base 64, XLerate, EWS64 S, TerraTV, DMX

This INI file is necessary because new driver releases might require changes in the volume control configuration. Always use the TerraTec drivers!

This error code can only be returned if the initial hardware detection failed.

RT (RDS group type 2)

PS (RDS group type 0)

CT (RDS group type 4)