TerraTec

ActiveRadio

Hardware Interface

Version 1.1

Author: Friedhelm Birth

Date: tt.07.jj

Contents

1	Introduction	2

1.1	ActiveRadio variations	2

1.2	Short description	2

2	Pin configuration	3

2.1	ActiveRadio Upgrade	3

2.1.1	The Radio Connector	3

2.1.2	Control Port bits	3

2.1.3	Special case: DMX	4

2.1.4	Special case: TerraTV+	4

2.2	ActiveRadio ISA Standalone	5

3	Volume Control	6

3.1	ISA Standalone card	6

3.2	Upgrade module on a TerraTec Soundcard	6

3.3	Upgrade module on a TerraTV+	6

4	Plug and Play Requirements	7

4.1	How to determine the ActiveRadio PnP resources	7

4.1.1	Windows 95 Configuration Manager	7

4.1.2	Low level ISA PnP and PCI configuration routines	7

4.2	ISA Standalone card	8

4.3	Upgrade module on a Base 1 or Base 64	8

4.4	Upgrade module on an EWS64 S	8

4.5	Upgrade module on a XLerate	9

4.6	Upgrade module on an DMX	9

4.7	Upgrade module on a TerraTV	9

4.8	TerraTV Radio+	10

5	Application notes	11

5.1	OM5610 Matchbox global FM tuner	11

5.1.1	Data Sheet Correction	11

5.2	SAA6588 RDS/RBDS pre-processor	11

5.3	FM1216	11

Bibliography	12

# Introduction

This document describes the hardware interface of the ActiveRadio module. The information given here is restricted to issues specific for the TerraTec ActiveRadio cards. All issues regarding the control of the tuner and the RDS pre-processor must be taken from the data sheets listed in the Appendix.

## ActiveRadio variations

The ActiveRadio is available in two constructions:

- ActiveRadio ISA Standalone card

- ActiveRadio Upgrade module

The ActiveRadio Upgrade module can be connected with the TerraTec Radio Connector to one of the following TerraTec products:

- Base 1

- Base 64

- XLerate

- DMX

- EWS 64 S

- TerraTV+

In addition to above devices the TerraTV Radio+ has FM & RDS receiver functionality. The ActiveRadio software and the API bothj work with this card as well.

## Short description

The key components of the ActiveRadio are the PHILIPS OM5610 global matchbox tuner and the PHILIPS SAA6588 RDS/RBDS pre-processor.

The OM5610 is an integrated FM stereo radio circuit including digital tuning and control functions. Communication with the OM5610 is done via a three wire protocol.

The SAA6588 is a CMOS device that integrates all RDS/RBDS relevant functions in one chip. The pre-processed RDS/RBDS information is available via IIC-bus.

For the ISA standalone version volume control is implemented utilising a shift register and a simple resistor circuit.

The ActiveRadio doesn’t provide an interrupt to signal when new RDS data are avilable. This was done because interrupts are a very limited resource in a PC.  However polling the RDS data can be accomplished without much perfomance loss.

The TerraTV Radio+ uses the combined FM/TV tuner FM1216 instead of the OM5610. Note that TV and FM reception are excluding each other.

# Pin configuration

## ActiveRadio Upgrade

### The Radio Connector

The ActiveRadio connector is used to plug the upgrade module to a TerraTec sound card (or TerraTV). The connector consists of 20 pins.

| 19 | 17 | 15 | 13 | 11 | 9 | 7 | 5 | 3 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RES | RES | GND | SCL | CLCK | STEREO | RES | IOW# | GND | RO |
| RES | CS# | SDA | DATA | WREN | DAVN | IOR# | +12V | +5V | LO |
| 20 | 18 | 16 | 14 | 12 | 10 | 8 | 6 | 4 | 2 |

Table 2-1: Radio connector (data pins are bold)

RES 	– reserve

RO 	– radio audio out right

LO 	– radio audio out left

### Control Port bits

The tuner chip (OM5610) has a bus which consists of three wires (CLCK, DATA, WREN). These three signals, together with the STEREO pin communicate with the microcontroller.

The RDS decoder chip (SAA6588) offers an IIC interface with wires SDA and SCL. An additional wire DAVN is supplied to check if new RDS data are available.

| First I/O port | Pin name | Device | Direction | Function |
| --- | --- | --- | --- | --- |
| 0x01 | SDA | SAA6588 | I/O | IIC data |
| 0x02 | SCL | SAA6588 | O | IIC clock |
| 0x04 | DATA | OM5610 | I/O | Data input / output |
| 0x08 | CLCK | OM5610 | O | Clock input |
| 0x10 | WREN | OM5610 | O | Write / Read enable |
| 0x20 | STEREO | OM5610 | I | Stereo / AFC indicator |
| 0x40 | DAVN | SAA6588 | I | Data available indicator |
| 0x80 | RESERVE | --- | --- | Not used |

Table 2-2: Data bits of the ActiveRadio Control Port

### Special case: DMX

Since the DMX is a PCI card the ISA interface wires (IOR#, IOW# and CS#) on the Radio Connector are not served automatically.

To write a data byte to the ActiveRadio module, follow this order

- CS# 	 LOW

- IOW#	 LOW

- Write Data Byte (SDA, SCL, DATA, CLCK and/or WREN)

- IOW#-	 HIGH

- CS# 	 HIGH

To read:

- CS# 	 LOW

- IOR#	 LOW

- Read Data Byte (SDA, DATA, STEREO or DAVN)

- IOR#- 	 HIGH

- CS# 	 HIGH

| First I/O port | Pin name | Device | Direction | Function |
| --- | --- | --- | --- | --- |
| 0x0001 | SDA | SAA6588 | I/O | IIC data |
| 0x0002 | SCL | SAA6588 | O | IIC clock |
| 0x0004 | DATA | OM5610 | I/O | Data input / output |
| 0x0008 | CLCK | OM5610 | O | Clock input |
| 0x0010 | WREN | OM5610 | O | Write / Read enable |
| 0x0020 | STEREO | OM5610 | I | Stereo / AFC indicator |
| 0x0040 | DAVN | SAA6588 | I | Data available indicator |
| 0x0080 | RESERVE | --- | --- | Not used |
| 0x0100 | IOR# | ISA | O | Read enable |
| 0x0200 | IOW# | ISA | O | Write enable |
| 0x0400 | CS# | ISA | O | Chip select |

Table 2-3: DMX Control port bits

### Special case: TerraTV+

The TerraTV+ also is a PCI card and uses the same mechanism as described for the DMX. However the ISA interface pins are located differently:

| First I/O port | Pin name | Device | Direction | Function |
| --- | --- | --- | --- | --- |
| 0x0001 | SDA | SAA6588 | I/O | IIC data |
| 0x0002 | SCL | SAA6588 | O | IIC clock |
| 0x0004 | DATA | OM5610 | I/O | Data input / output |
| 0x0008 | CLCK | OM5610 | O | Clock input |
| 0x0010 | WREN | OM5610 | O | Write / Read enable |
| 0x0020 | STEREO | OM5610 | I | Stereo / AFC indicator |
| 0x0040 | DAVN | SAA6588 | I | Data available indicator |
| 0x0080 | RESERVE | --- | --- | Not used |
| 0x0100 | IOW# | ISA | O | Write enable |
| 0x0200 | IOR# | ISA | O | Read enable |
| 0x0400 | CS# | ISA | O | Chip select |

Table 2-4: TerraTV+ Control port bits

## ActiveRadio ISA Standalone

The ISA standalone card requires two additional wires for volume control. Therefore two I/O port addresses are necessary.

Writing on the second port triggers a CS (Chis select) which is used as the clock signal of a shift register. This shift register sequentielly receives a volume value and selects a resistor combination with its output.

Detailed information on how to set the volume on the ISA standalone card can be found in the chapter Volume Control.

| First I/O port | Pin name | Device | Direction | Function |
| --- | --- | --- | --- | --- |
| 0x01 | SDA | SAA6588 | I/O | IIC data |
| 0x02 | SCL | SAA6588 | O | IIC clock |
| 0x04 | DATA | OM5610 | I/O | Data input / output |
| 0x08 | CLCK | OM5610 | O | Clock input |
| 0x10 | WREN | OM5610 | O | Write / Read enable |
| 0x20 | STEREO | OM5610 | I | Stereo / AFC indicator |
| 0x40 | DAVN | SAA6588 | I | Data available indicator |
| 0x80 | VOLDATA | Volume | O | Volume data |

Table 2-5: Data bits of the ActiveRadio ISA Standalone Control Port

# Volume Control

## ISA Standalone card

Using two wires (VOLCLCK and VOLDATA) the volume value is written to an 8 bit shift register. The output of the shift register selects a resistor combination which is responsible for the audio output level.

Both channels (left and right) can be controlled separately. The value range is from 0 to 7.

| MSB |  |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R2 | R1 | R0 | Rx | Lx | L2 | L1 | L0 |

Table 3-6: Volume shift register:

Rx, Lx - not used; R2, L2 - right / left channel MSB

Data shift on rising edge of CLCK2. The MSB must be written first!

#define MASK_VOLDATA	0x80

void SetHWVolume (unsigned short VolPort,   	// second I/O port

unsigned char  Volume)	// Volume in range 0..7

{

int	  i;

Volume = Volume + (Volume * 32);

for (i=0;i<8;i++)

{

if (Volume & (0x80>>i))

_outp(VolPort,MASK_VOLDATA);	// write to 2nd port

else  _outp(VolPort,0x00);

}

}

## Upgrade module on a TerraTec Soundcard

The Audio Out  of the ActiveRadio module is connected to the soundcard CoDec via the Radio Connector.

The drivers for every supported card reserve a mixer line in the Windows Sound System mixer for the radio. You should use the mixer API (MMSYSTEM.H) to control this line.

## Upgrade module on a TerraTV+

Volume control via hardware is not possible for the TerraTV+. Only Mute On/Off is available via an exported DLL function.

Volume control via the Windows mixer is difficult because the TerraTV+’s Line Out can be connected to any soundcard.

# Plug and Play Requirements

The ActiveRadio ISA Standalone card is a Plug and Play device and requires two 8-bit port address.

The ActiveRadio Upgrade module requires one 8-bit port address. The port address depends on the PnP configuration of the carrier board (sound card or TerraTV+).

No other resources are used for the ActiveRadio. No interrupt is required.

## How to determine the ActiveRadio PnP resources

There are basically two ways to determine the installed hardware and the relevant port addresses.

### Windows 95 Configuration Manager

Get the current configuration via the Windows Configuration Manager:

The configuration manager routines are accessible either from a Virtual Device Driver (VxD) or from a 16-bit user-mode application. For further information see the Windows 95 DDK.

### Low level ISA PnP and PCI configuration routines

For detailed information see the literature listed in the bibliography.

## ISA Standalone card

| Bus | ISA |
| --- | --- |
| Vendor ID | TER2111 |
| Long Device ID | TER2111_DEV0000 |
| Short Device ID | TER2111 |
| Required ports | I/O port no. 0 and 1 of device no. 0 |
| Possible addresses for the first I/O port (Default is bold) | 0x590, 0x598, 0x5A0, 0x5A8, 0x5B0,0x5B8, 0x5C0, 0x5C8 |
| Possible addresses for the second I/O port | 0x591, 0x599, 0x5A1, 0x5A9, 0x5B1,0x5B9, 0x5C1, 0x5C9 |

Table 4-7: PnP data (ISA standalone)

## Upgrade module on a Base 1 or Base 64

| Bus | ISA |
| --- | --- |
| Audio Controller | Analog Devices AD1816 |
| Vendor ID | TER1411 |
| Long Device ID | TER1411_DEV0003 |
| Short Device ID | TER2211 |
| Required ports | I/O port no. 0 of device no. 3 |
| Possible addresses for the I/O port (Default is bold) | 0x590, 0x598, 0x5A0, 0x5A8, 0x5B0,0x5B8, 0x5C0, 0x5C8 |

Table 4-8: PnP data (Base 1/64)

## Upgrade module on an EWS64 S

| Bus | ISA |
| --- | --- |
| Audio Controller | Analog Devices AD1816 |
| Vendor ID | TER1112 |
| Long Device ID | TER1112_DEV0003 |
| Short Device ID | TER1103 |
| Required port | I/O port no. 1 of device no. 3 |
| Possible addresses for the I/O port (Default is bold) | 0x100, 0x102, ..., 0x32A, ..., 0x3FE |

Table 4-9: PnP data (EWS64 S)

## Upgrade module on a XLerate

| Bus | PCI |
| --- | --- |
| Audio controller | Aureal Vortex |
| Vendor ID | 12EB |
| Device ID | 0001 |
| Subsystem ID |  |
| Required port | 2nd I/O port (BAR2) |

Table 4-10: PnP data (XLerate)

## Upgrade module on an DMX

| Bus | PCI |
| --- | --- |
| Audio controller | ESS Canyon 3D |
| Vendor ID | 125D |
| Device ID | 1978 |
| Subsystem ID | 153B1116 |
| Required port | 1st I/O port |

Table 4-11:PnP data (DMX)

## Upgrade module on a TerraTV+

| Bus | PCI |
| --- | --- |
| Controller | Conexant BT848 or BT878 |
| Vendor ID | 109E |
| DeviceID | 0350 (BT848) 036E (BT878) |
| Subsystem ID | 00000000 ( BT848, Rev. 1.0 ) 153B1127 ( BT848, Rev. 1.0 ) 153B1127 ( BT878, Rev. 1.0) 18521852 ( BT878, Rev. 1.1 ) |
| Required port | 1st I/O port |

Table 4-12: PnP data (TerraTV+)

## TerraTV Radio+

| Bus | PCI |
| --- | --- |
| Controller | Conexant BT878 |
| Vendor ID | 109E |
| DeviceID | 036E |
| Subsystem ID | 153B1123 |
| Required port | 1st I/O port |

Table 4-13: PnP data (TerraTV Radio+)

# Application notes

## OM5610 Matchbox global FM tuner

### Data Sheet Correction

There is a confirmed error in the data sheet in table 4, page 5. The correct values are

| S0.17 | S1.16 | Search Level FM (V) |
| --- | --- | --- |
| 0 | 0 | >15 |
| 1 | 0 | >35 |
| 0 | 1 | >75 |
| 1 | 1 | >300 |

Table 5-14: OM5610 search level

## SAA6588 RDS/RBDS pre-processor

... to be updated.

## FM1216

... to be updated.

Bibliography

System-Programmierung für Windows 95, Walter Oney, Microsoft Press, 1996

The Window NT Device Driver Book, Art Baker, Prentice Hall, 1997

Windows NT Device Driver Development, P.G. Viscarola & W.A. Mason, MacMillan Technical Publishing, 1999

Data sheets

OM5610 Matchbox global FM tuner, Philips Semiconductors

SAA6588 RDS/RBDS pre-processor, Philips Semiconductors

FM1216 Desktop video & FM radio module system B/G, Philips Semiconductors

Specifications

DIN EN50067, Spezikation des Radio-Daten-Systems (RDS), Beuth Verlag, Berlin, 1992

PCI Local Bus Specification Rev. 2.1s, PCI Special Interest Group, June 1995

PCI BIOS Specification Rev 2.1, PCI Special Interest Group, August 1994

Plug and Play ISA Specification, Version 1.0a, Intel Corporation / Microsoft Corporation, May 1994

On early versions of this card Rx was at MSB position. The shift value of 32 in the source code must be replaced by 16 for these cards.