TerraTec

ActiveRadio

Windows NT 4.0 Driver Reference

Version 1.1

Author: Friedhelm Birth

Date: tt.07.jj

Contents

1	Introduction	2

1.1	Why is a driver necessary?	2

1.2	Main features	2

1.3	Limitations	2

2	Driver installation	3

2.1	Registry entries	3

3	Communicating with the driver	4

3.1	Loading the driver	5

3.1.1	Example	5

3.2	Opening the driver	6

3.2.1	Example	6

3.3	Communication (Sending Control Codes)	7

3.3.1	DeviceIoControl	7

3.3.2	Example	8

3.4	Closing the driver	9

3.4.1	Example	9

3.5	Unloading the driver	10

3.5.1	Example	10

4	Reference	11

4.1	Structures	11

4.1.1	OM5610READ_RETURN	11

4.1.2	RADIO_CONFIG	12

4.2	IOCTL Control Codes	13

4.2.1	IOCTL_FIND_RADIO	13

4.2.2	IOCTL_SET_PORT	14

4.2.3	IOCTL_SET_VOLPORT (ISA  Standalone only)	14

4.2.4	IOCTL_OM5610_SET_FREQUENCY	15

4.2.5	IOCTL_OM5610_GET_FREQUENCY	15

4.2.6	IOCTL_OM5610_GET_STATE	16

4.2.7	IOCTL_OM5610_SEARCH	16

4.2.8	IOCTL_SAA6588_WRITE_IIC	17

4.2.9	IOCTL_SAA6588_READ_IIC	18

4.2.10	IOCTL_SA_SET_VOLUME (ISA Standalone only)	19

Table of tables	20

Bibliography	21

# Introduction

This document gives information on the Windows NT 4.0 driver interface for the ActiveRadio. It describes how an application can communicate with the driver and gives a reference of the available IOCTL codes. Sample code is included.

It does not describe the ActiveRadio hardware and the details of the protocols for the tuner and the RDS/RBDS  pre-processor. This information can be found in the “ActiveRadio Hardware Interface” documentation and the data sheets listed in the Appendix.

Note: The ActiveRadio exists in two hardware versions: as an ISA Standalone card and as an upgrade module that can be connected to one of these TerraTec products: Base 1/ Base 64, Xlerate, EWS64 S or TerraTV. Some driver features only apply to one of these versions.

## Why is a driver necessary?

Windows NT 4.0 doesn’t allow direct port access from ring 3 code (applications). Port access is only allowed from ring 0 code. Therefore every hardware requires a kernel-mode driver for control.

TerraTec provides such a driver to control the ActiveRadio device. The driver is named “TTRadio.sys”.

## Main features

This driver accomplishes the following tasks:

- Hardware identification

- OM5610 tuner control

- SAA6588 RDS/RBDS pre-preprocessor control and raw data acquirement

- Volume control (for the ActiveRadio ISA standalone card only)

Although this driver was originally designed for Windows NT 4.0 it also works with Windows NT 5.0 and Windows 98.

## Limitations

No INF file for driver installation is supplied.

The driver doesn’t support the NT PnP Enabler.

ISA Standalone only: If the PnP Enabler is installed, it will find the ActiveRadio. If you haven’t supplied an INF file always choose “Do not install a driver”.

# Driver installation

Before you can use the driver it must be installed on your system.

There are two ways to install the driver:

- Provide an INF file

- Install the driver during Software Setup

The driver installation must include the following steps:

- Copy TTRADIO.SYS to C:\WINNT\SYSTEM32\DRIVERS. (C:\WINNT is your assumed NT directory).

- Write the driver information to the registry.

- Reboot

After these steps you can open the driver fromout your application and send driver control codes.

## Registry entries

To make Windows NT load the ActiveRadio driver create the branch

HKEY_LOCAL_MACHINE\System\CurrentControlSet\Services\TTRadio

and add the keys and their values listed in the table below.

| Key | Value | Explanation |
| --- | --- | --- |
| ErrorControl | 1 | Normal |
| Group | “Extended Base” | Device group |
| Start | 2 | Auto Load (driver is loaded by the Service Control Manager) |
| Type | 1 | Device type |
| DisplayName | “ActiveRadio device” | Name shown in the devices panel |

Table 2-1: Registry entries

You can also set Start = 4. Then the driver is not started at boot time but can be loaded manually (see chapter 2. Driver installation).

# Communicating with the driver

The communication with a NT driver comprises basically five steps:

- Loading the driver (“Start the service” in NT terminology)

- Opening the driver

- Communicating with the driver

- Closing the driver

- Unloading the driver

If the driver was loaded at boot time (see chapter 2), steps 1 and 5 are not necessary

The steps are described in detail in the next sections.

Note: For the Service Control Manager routines include WINSVC.H in your source files.

## Loading the driver

Usually the driver is loaded at boot time unless you set the start mode to 4 = MANUAL in the registry. In this case you have to load the driver manually in your application.

Note: NT’s security concept allows manual driver loading  only if you logged in in Administrator Mode. In User Mode the driver cannot be loaded!

Load the driver manually with the functions OpenSCManager, OpenService, StartService and CloseServiceHandle.

### Example

BOOL LoadDriver ()

{

SC_HANDLE hManager, hService;

// Open Service Control manager

hManager = OpenSCManager(NULL,NULL,GENERIC_READ|GENERIC_WRITE|GENERIC_EXECUTE);

if (hManager == NULL )  return(FALSE);		// ERROR !

hService =OpenService(hManager, "TTRadio",GENERIC_READ|GENERIC_WRITE|GENERIC_EXECUTE);

if (hService == NULL )

{

CloseServiceHandle(hManager);

return(FALSE);					// ERROR !

}

if ( !StartService(hService, 0, NULL) )  		// Start the service

{

CloseServiceHandle(hManager);

CloseServiceHandle(hService);

return(FALSE); 					// ERROR !

}

CloseServiceHandle(hManager);       		// Close out handles

CloseServiceHandle(hService);

return(TRUE);						// SUCCESS !

}

## Opening the driver

The driver is opened with the function CreateFile. Store the returned driver handle for further communcation with the driver.

### Example

HANDLE hDriver = CreateFile(

"\\\\.\\TTRADIO",           	// Open the Device "file"

GENERIC_WRITE|GENERIC_READ,

FILE_SHARE_WRITE|FILE_SHARE_READ,

NULL,

OPEN_EXISTING,

0,

NULL);

if (hDriver == INVALID_HANDLE_VALUE)        		// Was the device opened?

return FALSE;						// ERROR !

## Communication (Sending Control Codes)

The DeviceIoControl function sends a control code directly to a specified device driver, causing the corresponding device to perform the specified operation.

### DeviceIoControl

BOOL DeviceIoControl(

HANDLE  hDevice,         	// handle to device of interest

DWORD   dwIoControlCode, 	// control code of operation to perform

LPVOID  lpInBuffer,      	// pointer to buffer to supply input data

DWORD   nInBufferSize,   	// size of input buffer

LPVOID  lpOutBuffer,     	// pointer to buffer to receive output data

DWORD   nOutBufferSize,  	// size of output buffer

LPDWORD lpBytesReturned,  	// pointer to variable to receive output

// byte count

LPOVERLAPPED lpOverlapped   	// pointer to overlapped structure for

// asynchronous operation

);

#### Parameters

hDevice

Handle to the device that is to perform the operation. Returned by the CreateFile function.

dwIoControlCode

Specifies the control code for the operation.  The supported control codes are listed in chapter 4.2.

lpInBuffer

Pointer to a buffer that contains the data required to perform the operation.

This parameter can be NULL if the dwIoControlCode parameter specifies an operation that does not require input data.

nInBufferSize

Specifies the size, in bytes, of the buffer pointed to by lpInBuffer.

lpOutBuffer

Pointer to a buffer that receives the operation's output data.

This parameter can be NULL if the dwIoControlCode parameter specifies an operation that does not produce output data.

nOutBufferSize

Specifies the size, in bytes, of the buffer pointed to by lpOutBuffer.

lpBytesReturned

Pointer to a variable that receives the size, in bytes, of the data stored into the buffer pointed to by lpOutBuffer.

lpOverlapped

Always NULL in our case.

#### Return Values

If the function succeeds, the return value is nonzero.

If the function fails, the return value is zero. To get extended error information, call GetLastError .

### Example

HANDLE hDriver;

WORD	wState;

ULONG 	ReturnedLength;

BOOL  fRet = DeviceIoControl(hDriver,IOCTL_OM5610_GET_STATE,NULL,

0,&wState,sizeof(wState),&ReturnedLength,NULL);

## Closing the driver

Before the appliction is terminated the driver must be closed with the function CloseHandle.

### Example

CloseHandle(hDriver);

## Unloading the driver

If the driver was manually loaded it should be unloaded after use. Use the functions OpenSCManager, OpenService, ControlService and CloseServiceHandle.

This is not necessary if the driver was automatically loaded at boot time.

### Example

BOOL UnloadDriver()

{

SC_HANDLE hManager, hService;       		// For Service handles

SERVICE_STATUS ss;                  		// For call

hManager = OpenSCManager(NULL, NULL,GENERIC_READ|GENERIC_WRITE|GENERIC_EXECUTE);

if (hManager == NULL ) 	return(FALSE);	// ERROR !

hService = OpenService(hManager, "TTRadio",GENERIC_READ|GENERIC_WRITE|GENERIC_EXECUTE);

if (hService) == NULL )

{

CloseServiceHandle(hManager);

return(FALSE);				// ERROR !

}

if (!ControlService(hService, SERVICE_CONTROL_STOP, &ss) )

{

CloseServiceHandle(hManager);    	// Close out handles

CloseServiceHandle(hService);

return(FALSE);				// ERROR !

}

CloseServiceHandle(hManager);       		// Close out handles

CloseServiceHandle(hService);

return(TRUE);

}

# Reference

All required type definitions and IOCTL control codes are given in GPIOCTL.H.

Note: Only the structures and control codes in this reference are verified. Additional sections in GPIOCTL.H are preliminary and should not be used.

## Structures

### OM5610READ_RETURN

This structure is not used by a IOCTL control code but might be useful for keeping the OM5610 information together.

typedef struct tagOM5610READ_RETURN	// accesses the shift register either as

{   						// DWORD or bit field

ULONG		SR;

UCHAR		fStereo;		// 1 - stereo        , 0 - mono

UCHAR		fLock;			// 1 - AFC not locked, 0 - AFC locked

}OM5610READ_RETURN;

### RADIO_CONFIG

This structure is filled by the control code IOCTL_FIND_RADIO.

typedef struct tagRADIO_CONFIG

{

UCHAR		HW_ID;

Char		VendorID[8];

ULONG		RadioPort;

ULONG		VolPort;

}RADIO_CONFIG;

HW_ID

Each TerraTec product with a radio or radio-connector has a unique hardware ID.

VendorID

Each PnP card (ISA or PCI) has a Vendor ID to identify the manufacturer and product.

RadioPort

This item is filled with the first I/O address of the identified radio device. It is zero if the hardware identification failed. The first I/O address is used for tuner and RDS pre-processor control.

VolPort

This item is filled with the second I/O address of the identified radio device. It is zero if the hardware identification failed. The second I/O address is used for the volume control on the ISA Standalone card.

It is zero for all other cards.

| Hardware ID | Product | Vendor ID |
| --- | --- | --- |
| 0 | Reserved / Error | --- |
| 1 | Reserved | --- |
| 2 | ActiveRadio ISA Standalone | TER2111 |
| 3 | Base 1 / Base 64 | TER1411 |
| 4 | Reserved | --- |
| 5 | XLerate | 12EB |
| 6 | EWS64 S | TER1112 |
| 7 | TerraTV | 109E |

Table 4-2: Hardware and Vendor IDs

## IOCTL Control Codes

### IOCTL_FIND_RADIO

This control code causes the driver to check which ISA and PCI cards are installed. If a relevant card was found the driver sends an IIC command and tests RDS pre-processor response. If successful the driver uses the detected I/O addresses for radio control. If no radio device is found or a device is deactivated the value of RadioPort int the returnd structure is zero.

The hardware check is done in the following order:

- XLerate

- TerraTV

- ISA cards (ISA Standalone, Base 1/64, EWS64S)

If more than one radio is installed the first radio found is used.

The configuration information is returned in a RADIO_CONFIG structure. It is for informational purpose only. The driver uses the detected I/O addresses unless you set the addresses manual by sending IO_SET_PORT or IO_SET_VOLPORT control codes.

| Parameter | Value | Description |
| --- | --- | --- |
| hDevice | driver handle | returned by CreateFile |
| dwIoControlCode | IOCTL_FIND_RADIO |  |
| lpInBuffer | NULL |  |
| nInBufferSize | 0 |  |
| lpOutBuffer | pointer to RADIO_CONFIG structure | filled with configuration information |
| nOutBufferSize | sizeof (RADIO_CONFIG) |  |
| lpBytesReturned | PULONG pRetLen | sizeof (RADIO_CFG) if successful, 0 if failed |
| lpOverlapped | NULL |  |

Table 4-3: IOCTL_FIND_RADIO parameters

### IOCTL_SET_PORT

This control code allows you to manually set the radio control port. Usually you don’t have to use it because IOCTL_FIND_RADIO automatically configures the driver with the right port addresses.

Fill the input buffer with the new I/O port address before you call DeviceIoControl.

| Parameter | Value | Description |
| --- | --- | --- |
| hDevice | driver handle | returned by CreateFile |
| dwIoControlCode | IOCTL_SET_PORT |  |
| lpInBuffer | PULONG pRadioPort | points to the new I/O port address |
| nInBufferSize | sizeof (ULONG) |  |
| lpOutBuffer | NULL |  |
| nOutBufferSize | 0 |  |
| lpBytesReturned | PULONG pRetLen | sizeof (ULONG) if successful, 0 if failed |
| lpOverlapped | NULL |  |

Table 4-4: IOCTL_SET_PORT parameters

### IOCTL_SET_VOLPORT (ISA  Standalone only)

This control code allows you to manually set the volume control port. Usually you don’t have to use it because IOCTL_FIND_RADIO automatically configures the driver with the right port addresses.

The volume control port is only used for the ISA Standalone card. It is ignored for all other cards.

Fill the input buffer with the new I/O port address before you call DeviceIoControl.

| Parameter | Value | Description |
| --- | --- | --- |
| hDevice | driver handle | returned by CreateFile |
| dwIoControlCode | IOCTL_SET_VOLPORT |  |
| lpInBuffer | PULONG pVolPort | points to the new I/O port address |
| nInBufferSize | sizeof (ULONG) |  |
| lpOutBuffer | NULL |  |
| nOutBufferSize | 0 |  |
| lpBytesReturned | PULONG pRetLen | sizeof (ULONG) if successful, 0 if failed |
| lpOverlapped | NULL |  |

Table 4-5: IOCTL_SET_VOLPORT parameters

### IOCTL_OM5610_SET_FREQUENCY

The name of this control code might be misleading. IOCTL_OM5610_SET_FREQUENCY simplay writes the control register of the OM6510 tuner chip. You can use IOCTL_OM5610_SET_FREQUENCY  to set a preset frequency, switch stereo/mono and (theoretically) start a search (up or down). For the usage of the control register see the OM5610 data sheet.

| Parameter | Value | Description |
| --- | --- | --- |
| hDevice | driver handle | returned by CreateFile |
| dwIoControlCode | IOCTL_OM5610_SET_FREQUENCY |  |
| lpInBuffer | PULONG pCtrlReg | points to the new OM5610 control register content (only the LSB 24 bits are used) |
| nInBufferSize | sizeof (ULONG) |  |
| lpOutBuffer | NULL |  |
| nOutBufferSize | 0 |  |
| lpBytesReturned | PULONG pRetLen | sizeof (ULONG) if successful, 0 if failed |
| lpOverlapped | NULL |  |

Table 4-6: IOCTL_OM5610_SET_FREQUENCY parameters

### IOCTL_OM5610_GET_FREQUENCY

This control code is mainly used to read back the frequency after a search. It can also be used to read back the frequency after a new frequency was set. This might be useful because the tuning algorithm tunes automatically to the best frequency. This frequency might slightly differ from the preset value. But take care reading back the frequency is only possible directly after a IOCTL_OM5610_SET_FREQUENCY command. If you read the frequency again you will read zero.

| Parameter | Value | Description |
| --- | --- | --- |
| hDevice | driver handle | returned by CreateFile |
| dwIoControlCode | IOCTL_OM5610_GET_FREQUENCY |  |
| lpInBuffer | NULL | points to the new OM5610 control register content (only the LSB 24 bits are used) |
| nInBufferSize | 0 |  |
| lpOutBuffer | PULONG pCtrlReg | pCtrlReg is filled with the contents of the control register, incl. the frequency value |
| nOutBufferSize | sizeof (ULONG) |  |
| lpBytesReturned | PULONG pRetLen | sizeof (ULONG) if successful, 0 if failed |
| lpOverlapped | NULL |  |

Table 4-7: IOCTL_OM5610_GET_FREQUENCY parameters

### IOCTL_OM5610_GET_STATE

The OM5610 tuner supplies two status flags:

- Stereo reflects the current stations stereo mode (stereo or mono)

- AFC is set if the tuner locked to a frequency, zero otherwise

IOCTL_OM5610_GET_STATE returns these status values in a 16 bit word:

| LSB Byte | 1 - station is stereo | 0 - station is mono |
| --- | --- | --- |
| MSB Byte | 1 - tuner is locked | 0 - tuner is not locked |

Table 4-8: OM5610 status flags

| Parameter | Value | Description |
| --- | --- | --- |
| hDevice | driver handle | returned by CreateFile |
| dwIoControlCode | IOCTL_OM5610_GET_STATE |  |
| lpInBuffer | NULL |  |
| nInBufferSize | 0 |  |
| lpOutBuffer | PUSHORT pState | pState is filled with the values of the OM6510 status flags |
| nOutBufferSize | sizeof (USHORT) |  |
| lpBytesReturned | PULONG pRetLen | sizeof (USHORT) if successful, 0 if failed |
| lpOverlapped | NULL |  |

Table 4-9: IOCTL_OM5610_GET_STATE parameters

### IOCTL_OM5610_SEARCH

IOCTL_OM5610_SEARCH combines the IOCTL_OM5610_SET_FREQUENCY and IOCTL_OM5610_GET_FREQUENCY control codes. The difference is that it has a wait loop integrated to guarantee that the search is finished before the frequency is read back.

You have to set the bits for the search direction, the search sensitivity level and the Local/DX flags in the control register before this command.

| Parameter | Value | Description |
| --- | --- | --- |
| hDevice | driver handle | returned by CreateFile |
| dwIoControlCode | IOCTL_OM5610_SEARCH |  |
| lpInBuffer | PULONG pCtrlReg | points to the control register content filled with the search direction, level and Local/DX flag |
| nInBufferSize | sizeof (PULONG) |  |
| lpOutBuffer | PULONG pCtrlReg | points to the read back control register content with the new frequency value |
| nOutBufferSize | sizeof (PULONG) |  |
| lpBytesReturned | PULONG pRetLen | sizeof (ULONG) if successful, 0 if failed |
| lpOverlapped | NULL |  |

Table 4-10: IOCTL_OM5610_SEARCH parameters

### IOCTL_SAA6588_WRITE_IIC

This control code sends an IIC command sequence to the SAA6588 RDS/RBDS pre.processor.

It usually is used only once to initialize the decoder. You can also use it to resynchronize the decoder manually e.g. when you set a new frequency.

The input buffer IICData is an array of three bytes. In the following table the corresponding SAA6588 register are given.

| IICData[0] | RDCONT | initialization and mode detection |
| --- | --- | --- |
| IICData[1] | RDPSFL | pause detector ans synchronization flywheel setting |
| IICData[2] | RDPSSG | pause detector/oscillator frequency and signal quality |

Table 4-11: IIC input buffer bytes

| Parameter | Value | Description |
| --- | --- | --- |
| hDevice | driver handle | returned by CreateFile |
| dwIoControlCode | IOCTL_SAA6588_WRITE_IIC |  |
| lpInBuffer | PUCHAR pIICData | points to a buffer containing the IIC input registers |
| nInBufferSize | 3*sizeof (PUCHAR) |  |
| lpOutBuffer | NULL |  |
| nOutBufferSize | 0 |  |
| lpBytesReturned | PULONG pRetLen | 0 |
| lpOverlapped | NULL |  |

Table 4-12: IOCTL_SAA6588_WRITE_IIC parameters

### IOCTL_SAA6588_READ_IIC

This is the function used to gather RDS/RBDS data and decoder status information.

Since the ActiveRadio hardware doesn’t provide an interrupt you have to poll the SAA6588 with a period < 21ms. Windows offers a multimedia timer (timesetEvent) with a theoretical resolution of one millisecond. However for times < 20ms this actual resolution is not very exact. To be sure not to miss any blocks you should poll even more frequently than every 21 ms and check if the current block was already read.

This control code returns a seven byte buffer:

| IICData[0] | RDSTAT | decoder and RDS/RBDS data status information |
| --- | --- | --- |
| IICData[1] | RDLBLH | last processed blick (HIGH byte) |
| IICData[2] | RDLBLL | last processed blick (LOW byte) |
| IICData[3] | RDPBLH | previous processed blick (HIGH byte) |
| IICData[4] | RDPBLL | previous processed blick (LOW byte) |
| IICData[5] | RDESTA | error status information |
| IICData[6] | RDSGQL | signal quality indication |

Table 4-13: IIC output buffer bytes

| Parameter | Value | Description |
| --- | --- | --- |
| hDevice | driver handle | returned by CreateFile |
| dwIoControlCode | IOCTL_SAA6588_READ_IIC |  |
| lpInBuffer | NULL |  |
| nInBufferSize | 0 |  |
| lpOutBuffer | PUCHAR pIICData | points to buffer receiving the SAA6588 output registers |
| nOutBufferSize | 7*sizeof (PUCHAR) |  |
| lpBytesReturned | PULONG pRetLen | nOutBufferSize if successful, 0 if failed |
| lpOverlapped | NULL |  |

Table 4-14: IOCTL_SAA6588_READ_IIC parameters

### IOCTL_SA_SET_VOLUME (ISA Standalone only)

This control code applies only to the ISA Standalone card. The ISA Standalone card is the only card with a volume control on board.

The volume for the Upgrade module can only be set on the carrier board .

The volume for the ISA Standalone card can range from 0..7

| Parameter | Value | Description |
| --- | --- | --- |
| hDevice | driver handle | returned by CreateFile |
| dwIoControlCode | IOCTL_SA_SET_VOLUME |  |
| lpInBuffer | PUCHAR pVolume | points to volume value (0..7) |
| nInBufferSize | sizeof (PUCHAR) |  |
| lpOutBuffer | NULL |  |
| nOutBufferSize | 0 |  |
| lpBytesReturned | 0 |  |
| lpOverlapped | NULL |  |

Table 4-15: IOCTL_SA_SET_VOLUME parameters

Table of tables

Table 2-1: Registry entries	3

Table 4-1: Hardware and Vendor IDs	12

Table 4-2: IOCTL_FIND_RADIO parameters	13

Table 4-3: IOCTL_SET_PORT parameters	14

Table 4-4: IOCTL_SET_VOLPORT parameters	14

Table 4-5: IOCTL_OM5610_SET_FREQUENCY parameters	15

Table 4-6: IOCTL_OM5610_GET_FREQUENCY parameters	15

Table 4-7: OM5610 status flags	16

Table 4-8: IOCTL_OM5610_GET_STATE parameters	16

Table 4-9: IOCTL_OM5610_SEARCH parameters	16

Table 4-10: IIC input buffer bytes	17

Table 4-11: IOCTL_SAA6588_WRITE_IIC parameters	17

Table 4-12: IIC output buffer bytes	18

Table 4-13: IOCTL_SAA6588_READ_IIC parameters	18

Table 4-14: IOCTL_SA_SET_VOLUME parameters	19

Bibliography

Microsoft Windows NT 4.0 DDK

The Windows NT Device Driver Book, Art Baker, Prentice-Hall, 1997

Data sheets

OM5610 Matchbox global FM tuner, Philips Semiconductors

SAA6588 RDS/RBDS pre-processor, Philips Semiconductors

‘Relevant card’ means either a ISA standalone card or a TerraTec card with radio connector.

For a search better use IOCTL_OM5610_SEARCH

It is possible to use the same buffer for input and output

If the carrier board is a sound card the volume can be set via the Windows Sound system. Mixer functions can be found in MMSYSTEM.H.